# How to list Ethernet Adapters’ information

If user don’t know the settings for Ethernet Adapters connected to Local Area Network, user can get the settings using NetJetManager program.. Open this program, it will appear a window as follows:

- Execute menu Manage>Broadcast to local network to get the settings for Ethernet Adapters which IP address is in the same sub-net with PC’s IP address. For example, assuming PC’s IP address is 192.168.1.10, its sub-net mask is 255.255.255.0, one Ethernet Adapter’s IP address is 192.168.1.252, another Ethernet Adapter’s IP address is 192.168.2.20(Assuming all Ethernet Adapters’ sub-net mask is 255.255.255.0), when execute this menu command, program will list the detailed information of the first Ethernet Adapter(IP is 192.168.1.252),and another Ethernet Adapter will not be listed.

- Manage>Broadcast to other network – If user want program to list the other Ethernet Adapter as above example(IP is 192.168.2.20), execute this menu command, it will display a dialog as follows:

Type 192.168.2.X in IP Address, for example 192.168.2.3, type 255.255.255.0 in Subnet mask, and then press Ok button, program will list the second Ethernet Adapter(its IP is 192.168.2.20).

Note - If user want to use this function, the PC with NetJetManager software must can access the 192.168.2.X sub-net by gateway.

# Introduction of parameters

- Device Serial – This ID is distinguish itself from other Ethernet Adapters, and this ID will be stick on outline border of Ethernet Adapters.

- Device Name – Not used.

- Local IP – The Ethernet Adapter’s IP address.

- Destination IP – Not used.

- Port – It is IP port. This value must be set to same with E2000TP.exe. Please refer to E2000PCmanual to see how to set the IP port.

- Mode – It can be two modes: server and client. Here is fixed to server.

If user want to learn about more detailed information, for example, baud rate, etc., please continue these operations as follows:

First, move mouse to one item about which you want to learn.

Then, double-click it, it will display a dialog as follows:

This dialog list all information that stored in Ethernet Adapter. Please refer to NetJetSetting manual to learn about the meaning.

User can change the parameters as above and then click Setup to set these parameters to Ethernet Adapter.