E1000 Series

MOVING SIGN

OPERATION MANUAL

TABLE OF CONTENTS

INTRODUCTION	3

REMOTE CONTROL PROGRAMMING	3

QUICKSTART PROGRAMMING	4

RUNNING A SAVED MESSAGE	5

RECALLING A SAVED MESSAGE FOR EDIT	5

MORE INTRICATE EXAMPLES	5

PROGRAMMING A SEQUENCE FILE	7

DELETE MESSAGE FILE	8

PASSWORD PROTECTION	8

USING THE INTERNATIONAL CHARACTER SET	9

GETTING A GRAPHIC FOR DISPLAY	9

EDIT THE USER DEFINED GRAPHICS	9

COMMUNICATION WITH PC	9

SELECT THE DATE FORMAT	10

APPENDIX A: ADDITIONAL PROGRAMMING TIPS	11

APPENDIX B: TIMER OPTIONS	13

APPENDIX C: LISTING OF COLOR	14

APPENDIX D: SPECIFICATION AND FEATURE SUMMARY	15

APPENDIX E: LISTING OF SYMBOL CHARACTERS	16

APPENDIX F: MECHANICAL SPECIFICATIONS	16

APPENDIX G: INTERNATIONAL CHARACTERS	17

APPENDIX H: FILE LOG	18

APPENDIX I: REMOTE KEYS DESCRIPTION	19

APPENDIX J: ORDERING INFORMATION	22

TERMS AND CONDITIONS	23

TROUBLE SHOOTING	24

IMPORTANT SAFETY PRECAUTIONS

- Avoid locations that are subject to direct sunlight(except outdoor models), excessive heat, moisture and dust.

- Do not place heavy objects or spill liquids on the sign.

- Unplug this product from the wall outlet before cleaning, Do not use liquid cleaners or aerosol cleaners. Use a damp cloth for cleaning.

- This product should be operated from the type of power source indicated on the marking label. If you are not sure of power available, consult your dealer or local power company.

- Unplug this product from the wall outlet and refer servicing to qualified service personnel under the following conditions.

- When the power cord or plug is damaged or frayed.

- If liquid has been spilled into the product.

- If the product has been exposed to rain or water.

- If the product has been dropped or the cabinet has been damaged.

# INTRODUCTION

The E1000 series moving sign is a one-line (7 points height), ready-to-run instant moving message media. Easy to operate, the E1000 series color models features eight colors (a total of 16 color combinations when using colored backgrounds for your message) and the ability to store up to 100 messages that you create. Additionally, the E1000 series also has built-in:

- Eight pre-defined graph for easy to recall

- Thirty graphic symbols such as phone, car, sunglasses, etc. for use in place of spelling out a word

- Twenty-four methods of displaying your message (scroll up from bottom to top, opened from center, explode, etc.)

- Eight display speeds

- Three different audible beep sounds can be added anywhere within the message

- The ability for your message to pause for up to eight seconds

- The ability to automatically power on or turn off the display

- The ability to display multiple message files in a defined sequence

- The ability to display both the current date and time within your message

NOTE: the E1xx1 models support outdoor application and the E1xx2 models can even display the current temperature. Please refer to Appendix J: ORDERING INFORMATION.

The E1000 series is designed for both indoor and outdoor use and includes mounting device to mount the unit on ceiling, on a wall or as legs for the unit to remain freestanding on a table.

The power for the sign is supplied by the included power transformer, before you plug the power plug into the electrical outlet, make sure the voltage marking on the power transformer matches that of the electrical outlet.

You should mount the E1000 series within close proximity to an outlet and preferably where sunlight will not directly fall on the display (except outdoor models).

# REMOTE CONTROL PROGRAMMING

Included with the E1000 series is a remote control for programming your messages into the unit. The remote control operates using infrared technology, which frees you from unnecessary wires and having to be at the unit in order to program a message change. When programming, you should aim the remote control at the sign unit in order to ensure that the sign receives your programming. Because each character you type is automatically sent to the unit, an audible beep is sounded to verify that the sign received the character or program code.

When certain keys are pressed, a control signal is sent to the message sign (i.e. color change or letter style change) due to the space on the sign, the control signal may not be shown in exactly wording. The following are control codes that appear based upon the key pressed.

| METHOD | [CYCLIC] | [IMMED] | [OPEN←] | [OPEN→] | [OPEN<>] |
| --- | --- | --- | --- | --- | --- |
|  | [OPEN><] | [COVER<>] | [COVER←] | [COVER→] | [COVET><] |
|  | [SCROLL↑] | [SCROLL↓] | [INTER><] | [INCOVER><] | [COVER↑] |
|  | [COVER↓] | [SCANLN] | [EXPLODE] | [PACMAN] | [FALL] |
|  | [SHOOT] | [FLASH] | [RANDOM] | [SLIDE IN] |  |

*Please note if the display message is longer than one page, whatever the display method used, the display method will be [OPEN←].

FONT	[5×6] [5×11] [7×6] [7×11] [7×9] [7×17] [Small]

SPEED	[SPEED 1] [SPEED 2] [SPEED 3] [SPEED 4] [SPEED 5] [SPEED 6] [SPEED 7]

[SPEED 8]

GRAPHICS	*The graphics will de displayed as what they are.

PHRASE	[ANIM 1] [ANIM 2] [ANIM 3] [ANIM 4] [ANIM 5] [ANIM 6] [ANIM 7] [ANIM 8]

COLOR	[RED] [ORA] [YEL] [GRE] [MIX]

SYMBOL	Refer to Appendix E for a list Graphic Symbol.

BEEP	[BEEP 1] [BEEP 2] [BEEP 3]

Using the function keys embeds the above program controls into your message to vary the speed of display, color, method of display, sound, symbols used and character styles. Using these special functions will add variety and excitement to your messages.

# QUICKSTART PROGRAMMING

### A SIMPLE MESSAGE IN FIVE EASY STEPS

The E1000 series stores up to 100 messages in numbered data files. These files are numbered from 00 to 99. File 00 is factory built and is read only. Appendix H is a log for you to use when you program a message and store it with its two-digit number.

A Simple Programming Example -

Note:	Underline denotes words or numbers to type, xxxx denotes key names. To correct a mistake, press the DEL key to remove it.

User can toggle the capital letter and small letter by pressing the key CAP, and use the ALT key to display symbol on key pad.

| Step | Keys to press | Explanation | Display |
| --- | --- | --- | --- |
| 1. | PROG | Begins the edit program mode | 00=>OPEN: ?? |
| 2. | 01 ENT | The first two digit file number (01-99) are allowed for naming message files [CYCLIC] is the default method of display code which means to cycle through at random | [CYCLIC] |
| 3. | WELCOME VISITORS | Type the text you wish to display in you message | WELCOME VISITORS |
| 4. | RUN | Ends the edit program mode | SAVE? (Y/N) |
| 5. | Y | Saves the message as data file number 01 | Message begins to be displayed |

You have just created your first message! Here are some important things that you learned from this first exercise:

- An audible beep is heard when the unit accepts programming input from the remote.

This is one additional way besides viewing the unit to assure you that the button you pressed has been received and understood.

- The programming of a message can be as simple or as complex as you desire you decide what you want your message to say and how best to display it .In the above example the message was only two words, but you could have typed a complete set of instructions.

- If you made mistakes while typing, you correct them by pressing the DEL key until you have removed the mistake(s) and re-entered in the message correctly.

- The default color for messages is bright Red. A list of all color possibilities can be found in Appendix C.

- You have the option at the end of your programming to save your message. If you only need to display a message for a single instance, simply select N to not save your message as numbered data file. Your message will still be displayed, but the next time you open up the numbered data file (in this case, file number 01) the message would not exist.

- E1000 series always begins the editing of a new message with the default method of display as [CYCLIC], which stands for cycle through all color and method possibilities at random.

You can follow the above procedures to enter some more messages, as long as the total file numbers not excess 99 and the total length of characters not excess 7000.

# RUNNING A SAVED MESSAGE

Normally, the sign will display the message which is the latest edit/modified, but there is an option that user can choose any message to display at any time.

A Run Message File Example -

| Step | Key to Press | Explanation | Display |
| --- | --- | --- | --- |
| 1. | RUN | Run a saved message | RUN : ?? |
| 2. | 02 ENT | Select file 02 | Message begins to be displayed |

# RECALLING A SAVED MESSAGE FOR EDIT

To recall a previously saved message, you again begin by pressing the PROG key, When the E1000 series prompts you for a file number, input the two-digit number and press the ENT key. If there is a previously stored message under that file number, the display will show [OLDFIL] [CYCLIC], or  may not be [CYCLIC] depending on actual content.

A Simple Message File Recall Example -

| Step | Key to press | Explanation | Display |
| --- | --- | --- | --- |
| 1. | PROG | Begins the edit program mode | 01=>OPEN : ?? |
| 2. | 01 ENT | This is the file number for the previously stored message file (see previous example) | OLDFIL [CYCLIC] |
| 3. | RUN | Ends the edit program mode | SAVE ? (Y/N) |
| 4. | N | There is no need to re-save the data file | Message begins to be displayed |

Every time you want to recall a previously saved message, follow the 4 steps listed above, substituting in the appropriate file number.

# MORE INTRICATE EXAMPLES

Once you have successfully programmed your first message, you find that using the remote to program your E1000 series is extremely easy. Adding spectacular color, animation and varying the shape of the letters is also easy to perform. And while it takes a little more time to tackle more intricate messages, they add sparkle and increase readability.

Because the E1000 series can display 16 different colors, and has various options selecting the method by which your messages appear on the sign, you can create extremely complex messages. There is no single method or pattern that you should start each message with - be creative in your approach to displaying your message and experiment with different color combinations, different methods for having the message appear (scroll from left to right versus immediately appearing, etc.).

##### A More Complex Programming Example -

We want to program in a message that is eye-catching and appealing to shoppers about our new Sporting Goods Department. Our message is simple: VISIT OUR NEW SPORTING GOODS DEPARTMENT TODAY. We’ll use color and variety to bring to life the message and attract some new sporting goods buyers.

| Step | Keys to press | Explanation | Display |
| --- | --- | --- | --- |
| 1. | PROG | Begins the edit program mode | 01=>OPEN : ?? |
| 2. | 02 ENT | This is the file number to assign to this new message file | [CYCLIC] |
| 3. | NEXT NEXT NEXT NEXT | Assigns the method of display to an Open From Center | [OPEN <>] |
| 4. | COL NEXT NEXT NEXT | Assigns the color Bright Orange | [ORA] |
| 5. | ###### VISIT OUR NEW | Enters in message text | VISIT OUR NEW |
| 6. | ###### COL NEXT NEXT NEXT NEXT NEXT NEXT NEXT NEXT | Assigns a rainbow color to test | [MIX] |
| 7. | MET NEXT NEXT NEXT NEXT NEXT | Changes display method to cover old text from Right | [COVER←] |
|  | NEXT NEXT |  |  |
| 8. | SPORTING GOODS DEPARTMENT | Enters message text | SPORTING GOODS DEPARTMENT |
| 9. | PAU NEXT | Pauses for 2 second delay at end of message before re-displaying it | [PAUSE 2] |
| 10. | RUN | Exits out of edit message mode | SAVE ? (Y/N) |
| 11. | Y | Save the data file as 02 | Message begins to be displayed |

That′s a pretty fancy message and still only eleven steps! The beauty of the E1000 series is that there is an unlimited number designs for your messages, just by changing the colors, methods of display, and the character sizes and shapes. You must decide what works best with your wall colors, room lighting etc.

Remarks:	The message “SPORTING GOODS DEPARTMENT” is more than one screen, the only display method is slide in from the right. User may try to shorten this message and try for other display method.

Refer to Appendix A to learn more about the functionality of the E1000 series and how best to use your sign to gain maximum benefits!

# PROGRAMMING A SEQUENCE FILE

The E1000 series has the option of displaying many files sequentially. You first create a sequence files with the file numbers of the messages you want to display. The following example leverages the power of sequencing to achieve desired results.

Sequence files have special names that begin with the letter ″S″ followed by a single digit number (S0-S9). As such, there are only ten sequence files which can be stored and retrieved by the E1000 series.

A Simple Sequence Example -

Let’s say that as the owner of a restaurant named Box Car Express. You have daily specials that appeal to the lunch crowd at noon. You have already created the following files:

| File Number | Message |
| --- | --- |
| 10 | Welcome to the Box Car Express |
| 11 | Today’s special is |
| 12 | 2-Pc. Chicken Meal with Mashed Potatoes and Gravy. |
| 13 | Our drink Special is |
| 14 | Diet Coke. |

You may want to display the above messages from 10:00 am to 1:00 pm, Monday to Friday. In order to display the entire message, your sequence file would contain the line:

10 11 12 13 14

| Step | Keys to press | Explanation | Display |
| --- | --- | --- | --- |
| 1. | PROG | Begins the edit message mode | 14=>OPEN : ?? |
| 2. | ###### S | For sequential file | [S0] |
| 3. | 1 ENT | Enter sequential file number | DAY 0 1 2 3 4 5 6 |
| 4. | → | Move right | 1 high lighted |
| 5. | NEXT / PREV | Toggle for ON/OFF | 1 in green with red background |
| 6. |  | Repeat step 4 & 5 for 2、3、4、5 until all 1、2、3、4、5 in green color |  |
| 7. | S | Set start time | START 00:00 |
| 8. | NEXT / PREV | Change the start timer | The digit in green will be changed |
| 9. | ← → | Use the arrow key for proper digit |  |
| 10 . |  | Repeat steps 8 & 9 until display shown 10:00 |  |
| 11. | S | Set stop time | END 23:59 |
| 12. |  | Repeat 8 & 9 until display shown 13:00 |  |
| 13. | ENT | Confirm the time setting | S1 |
| 14. | 10 11 12 13 14 | This is the sequence of file numbers to display, separated by a space. |  |
| 15. | RUN | Ends the edit program mode | SAVE ? (Y/N) |
| 16 | Y | Save the sequence file as S1 | Message begins to be displayed |

The real advantage of sequencing files is the ability to change only one segment of a long message! If you decide to change the meal special but not the drink special, you need only edit file number 12.

In essence, sequencing saves your time from having to edit long messages when only minor changes are needed.

Please be remained that once there is any sequence file and the time setting is valid, the sign will display the preset message starts from sequence file 00 through 09.

# DELETE MESSAGE FILE

In some circumstance, user may like to erase some of the message, the E1000 series has such an option for user to do so. After enter the Edit Mode, user can press the key CLR and cycling through the difference functions by pressing the PREV or NEXT key.

Listed below are the Erase functions and there usages:

DEL ALL		Clear all message files

DEL FILE ??		Delete the specified file

DEL MSG		Delete the message of the current opening file

DEL ON/OFF	Disable the Auto Turn on /Turn off timer function

DEL ALARM	Disable the alarm function

# PASSWORD PROTECTION

Password protection is built into the E1000 series Sign. In order to program a message under password protect mode, you must key in the correct password. You are required to enter the edit mode when you want to change password, reset password or disable password security.

##### Steps to change the password

| Step | Key to press | Explanation | Display |
| --- | --- | --- | --- |
| 1. | PROG | Enter Edit mode | 00=>OPEN : ?? |
| 2. | 00 ENT | Enter file 00 | OLDFIL [CYCLIC] |
| 3. | Press → while the ALT is being hold | Fast shift |  |
| 4. | Repeat step 3 | A few times until you reach to the end of the file, this is to make sure you will not modify the content of the file while doing password change |  |
| 5. | CPW | Enter password mode | UNUSE PASS |
| 6. | NEXT |  | USE PASS |
| 7. | NEXT |  | RES PASS |
| 8. | NEXT | Change password | CHANGE: *** |
| 9. | AAA ENT | Change the password to AAA | AGAIN: *** |
| 10. | AAA ENT | Confirm the password | Content of the data file |

Then you have changed the password to AAA.

Meaning of Display

UNUSE PASS		Turn off password security

USE PASS			Turn on password security

RES PASS			Reset to factory default password ″000″

CHANGE PASS		Change to new password

If for any reason the password is lost or forgotten, you must remove the SW2 jumper on the main board and then follow the ″Steps to change password″ from step 1 to step 7. In step 7, press enter. Thus the factory default password will be in effect (000). You may disable the password protection feature by leaving the SW2 jumper not connected.

# USING THE INTERNATIONAL CHARACTER SET

Using international characters in your messages is easy with the E1000 series. As you program your message, key in the corresponding English equivalent for the character and use the NEXT and PREV buttons to scro11 through the international characters that correspond to the English character (i. e. key in ″$″ and use the NEXT button until the ″£″ character appears if you want to display a number in British pound currency). If there is no corresponding English character first enter, key in a SPACE and then use the NEXT and PREV buttons to scroll through the list of available characters.

APPENDIX G shows all the international characters with the corresponding English characters.

# GETTING A GRAPHIC FOR DISPLAY

To get a graphic for display between any messages, the user should enter the Edit mode of the proper file, at the desired location, press GRA. User can cycle through the functions and press ENT when the display shown ″GET GRAPH″. All preset graphics will cycle through with the key PREV or NEXT pressed. Press ENT for the right one.

# EDIT THE USER DEFINED GRAPHICS

Inside the edit mode, user can make a new graphic or edit a user-defined graphic in the memory [GRAPHIC A] to [GRAPHIC H]. User can press the key GRA for choosing the appropriate function. After select the memory location, the display will show the user defined graphic (when enter the EDIT GRAPH function), or show a blank display only with a dot of cursor (when enter the MAKE GRAPH function). Edit the graph, user can use the direction keys (same key pad as D , E , F ,I, K, N, O, P ) to move the cursor onto the position and user can change the dot color by pressing PREV or NEXT key. If user want to draw a line, he can press the dot keys (same key pad as J ) and make a line through the cursor position. Turned off this draw line function by press the dot key again.

# COMMUNICATION WITH PC

User can use a personal computer to program the messages, then send it to the E1000s signs through RS-232 or RS-422 interface.

You can link up to 255 E1000s signs into one network controlled by one PC. Before the communication, you should press ESC in display mode, then choose SERIAL ADDRESS from the popup menu to set the sign’s serial address, from 1 to 255.

# SELECT THE DATE FORMAT

You should press ESC in display mode, and then choose DATE FORMAT from the popup menu to select a date format. Two date formats are available: dd-mm-yyyy and mm-dd-yyyy.

# APPENDIX A: ADDITIONAL PROGRAMMING TIPS

The following functions require that you move through the list of possibilities and make a selection. Use NEXT and PREV keys to move through the list of choices.

Function		Definition		Possibilities		Control

Code

METHOD		Display Option	Cyclic				[CYCLIC]

Immediate			[IMMED]

Open from Right		[OPEN←]

Open from Left		[OPEN→]

Open from center		[OPEN<>]

Open to Center		[OPEN><]

Cover from Center		[COVER<>]

Cover from Right		[COVER←]

Cover to Left			[COVER→]

Cover to Center		[COVER><]

Scroll Up			[SCROLL↑]

Scroll Down			[SCROLL↓]

Interlace to Center		[INTER><]

Interlace Cover		[INCOVER><]

Cover Up			[COVER↑]

Cover Down			[COVER↓]

Scan Line			[SCANLN]

Explode			[EXPLODE]

Pac Man			[PACMAN]

Fall & Stack			[FALL]

Shoot				[SHOOT]

Flash				[FLASH]

Random			[RANDOM]

Slide in			[SLIDE IN]

COLOR		Alters the color	see Appendix C

of text

FONT			Changes the shape	5×6 Short			[5×6]

and size of		5×11 Short & Wide		[5×11]

characters        	7×6 (Default)		[7×6]

7×11 Wide			[7×11]

7×9				[7×9]

7×17 Extra Wide		[7×17]

Small Fonts	[Small]

SPEED		Changes the		Speed 1 (Fastest)		[SPEED 1]

Scrolling  		Speed 2			[SPEED 2]

Speed of the display	Speed 3 			[SPEED 3]

Speed 4			[SPEED 4]

Speed 5			[SPEED 5]

Speed 6			[SPEED 6]

Speed 7                	[SPEED 7]

Speed 8 (Slowest)        	[SPEED 8]

GRAPHICS   Editable by user						[GRAPH A]

[GRAPH B]

[GRAPH C]

[GRAPH D]

[GRAPH E]

[GRAPH F]

[GRAPH G]

[GRAPH H]

GRAPHICS  Non-editable by	City                        	[GRAPH I]

user           	Car                         	[GRAPH J]

Vessel                       	[GRAPH K]

Tel                         	[GRAPH L]

Nature                      	[GRAPH M]

Ship                        	[GRAPH N]

Swim                       	[GRAPH O]

Cat                         	[GRAPH P]

PHRASE    Generates an		MERRY X`MAS        		[ANIM 1]

animated pre     	HAPPY NEW YEAR         	[ANIM 2]

programmed      	4TH JULY                    	[ANIM 3]

display          	HAPPY EASTER             	[ANIM 4]

HAPPY HALLOWEEN        	[ANIM 5]

DON`T DRINK & DRIVE       	[ANIM 6]

NO SMOKING               	[ANIM 7]

WELCOME               	[ANIM 8]

PAUSE    Generates a pause   	Pauses for 2 second delay       	[PAUSE 1]

within the		Pauses for 3 second delay		[PAUSE 2]

programmed        	Pauses for 4 second delay  		[PAUSE 3]

message          	Pauses for 6 second delay       	[PAUSE 4]

Pauses for 10 second delay      	[PAUSE 5]

Pauses for 20 second delay      	[PAUSE 6]

Pauses for 30 second delay      	[PAUSE 7]

Pauses for 60 second delay      	[PAUSE 8]

# APPENDIX B: TIMER OPTIONS

Time Options On The E1000 series

You have the option of programming the E1000 series to turn off the display at certain times during the day. Note that you may only set the timer functions while in the EDIT program mode of operation.

To select the timer function while in edit mode at the end of a message is fine, press the TIME key. Press the NEXT and PREV keys to cycle through the timer functions and press the ENT key to make your selection

To input times, press the Left and Right Arrow Keys to move through the digits and use the NEXT or PREV keys to increment or decrement the values for the digits.

| Function | Description |
| --- | --- |
| SET TIME | Adjust the Real Time Clock |
| SET DATE | Adjust the Date |
| ALARM HOUR | Set alarm times and interval |
| ALARM | Set the hourly alarm On/Off |
| 12H MODE | If selected, the time will be displayed in 12-hour format (Note: while programming, always enter the time in 24-hour format) |
| 24H MODE | If selected, the time will be displayed in 24-hour format |
| ADD TIME | Add a time display screen in the data file |
| ADD DATE | Add a date display screen in the data file |
| OFF TIME | Tums off the E1000 series at the programmed time of day |
| ON TIME | Tums on the E1000 series at the programmed time of day |

Additional Tips On Alarm Setting-

When you select the Alarm option, the sign will indicate C: XXXM: XXX.

Where XXX is a number from 000 to 255.

C is the total times of the alarm to be activated, and M is the time interval of each activation.

Example-

C: 123M: 001 - this means that there will be 123 times of alarm signal, and the time interval between each alarm signals will be 1 minute.

# APPENDIX C: LISTING OF COLOR

Listing of Colors-

| Color Name | Display Code |
| --- | --- |
| Red............................................................................................... ... Bright Red................... ................... ................... ................... ........ Orange................... ................... ................... ................... .............. Bright Orange................... ................... ................... ...................... Yellow................... ................... ................... ................... ............... Bright Yellow................... ................... ................... ...................... Green................... ................... ................... ................... ............... Bright Green................... ................... ................... ................... ..... Layer Mix (Rainbow) ................... ................... ................... .......... Bright Layer Mix (Rainbow) ................... ................... .................. Vertical Mix................... ................... ................... ................... ...... Saw Tooth Mix................... ................... ................... ................... . Green on Red................... ................... ................... ................... ... Red on Green................... ................... ................... ................... ... Orange on Red................... ................... ................... ................... . Yellow on Green................... ................... ................... ................... | [RED] [RED] * [ORA] [ORA] * [YEL] [YEL] * [GRE] [GRE] * [MIX] [MIX] * [MIX] [MIX] [MIX] [MIX] [MIX] [MIX] |

* The color of the dots will not change, except the brightness

# APPENDIX D: SPECIFICATION AND FEATURE SUMMARY

General Specifications-

1. Display Element	5×7 LED module

2. Display Color                        	8 colors: (for Color models)

Red, Green, Yellow, Orange, Light-Red,

Light-Green, Light Yellow and Light

Orange

3. Screen Matrix (H×W)               	7×80(dots) (for 1080C/R models)

4. Display Area (H×W)                	53mm×614mm (for 1080C/R models)

5. No. of Characters                   	13~16 (for 1080C/R models)

6. Character Set                       	English and European character

7. Operating Voltage                   	7.5V AC. (5V DC for 1xx3 models)

8. Power Adapter                      	Input: 120V/230V AC (not for 1xx3

models)

Output: 7.5V AC 36VA

9. Battery Back-up                        	60 days

10. Serial Communication                	RS-232C/RS-485

11. Real Time                           	24 hour or 12 hour display format

12. Color Combination                   	16 (for Color models)

13. Screen Function                     	24 effect

14. No. of Font                          	7 (Including Small Fonts)

15. Symbol Pattern                      	30 (refer to the Graphic symbol)

16. Operation Temperature               	0 to +45℃

17. Handheld Key Pad                   	Infra-red Remote Control

18. Sign Body                        	Composed by a black color housing and

with a	 brownish front display filter

Size	Length 70.5cm × Width 11.5cm ×

Thickness 5.5cm (for 1080C/R models)

19. Power Supply                        	Power transformer is mounted in a black

color casing

Size                                	Length 10cm × Width 6.5cm × Height

6cm (not for 1xx3 models)

20. Packing (Gift Box)	Include Sign Power and IR Remote Control

are placed in Gift Box

Size	Length 87cm × Width 17cm ×

Thickness 10cm (for 1080C/R models)

Main Features-

1.  Magic Key Feature	Single key programming to display the input

Character in sequence of change color and

font

2.  Message Channel	100 pages

3.  Memory Size	Approx. 7000 Characters

4.  Full Screen Graphic	8 Pre-program Graphic. 8 User Defined

Graphic. On screen edit

5.  Animation Graphic	8 pre-programmed animation graphic files.

(Refer to the Graphic and the Special key)

6.  Graphic Symbol	30 pre-programmed Graphic Symbol Pattern

7.  Message Display	Can be displayed in any desired page

sequence

8.  Speed control	8 smooth shift speed

9.  Networking	Support 128 locations and the locations are

Dip switch select

# APPENDIX E: LISTING OF SYMBOL CHARACTERS

Note: The actual symbol appear in brackets (“[  ]”) during the programming mode

| Sunny | Cloudy | Rainy | Clock |
| --- | --- | --- | --- |
| Phone | Glasses | Faucet | Rocket |
| Alien | Key | Sweater | Helicopter |
| Car | Tank | House | Teapot |
| Trees | Duck | Motorcycle | Bike |
| Crown | Hearts | Right Arrow | Left Arrow |
| Lower Left Arrow | Upper Left Arrow | Mug | Chair |
| Shoe | Martini Glass |  |  |

# APPENDIX F: MECHANICAL SPECIFICATIONS

- SIGN

Body is composed by a black color housing and a brownish front display filter.

SIZE: Length 70.5cm × Height 11.5cm × Thickness 5.5cm. (for 1080 models)

- POWER SUPPLY

Power transformer is mounted in a black color casing.

SIZE: Length 10cm × Height 6.5cm × Thickness 6cm. (for 1080 models)

- PACKING

Sign power supply and IR remote control are placed in a Gift Box.

SIZE: Length 87cm × Height 17cm × Thickness 10cm. (for 1080 models)

# APPENDIX G: INTERNATIONAL CHARACTERS

| English Character | International Characters |
| --- | --- |
| A | Ä Å Æ |
| C | Ç Ć |
| E | É |
| N | Ñ |
| O | Ö Ó |
| S | Ś |
| U | Ü |
| Z | Ż Ź |
| a | â ä à å æ á |
| c | ç |
| e | é ê ë è |
| i | ï î ì í |
| n | ñ ń |
| o | ô ö ò ó |
| t |  |
| u | ü û ù ú |
| y | ÿ |
| Space | α β Γ π Σ σ μ τ Φ θ Ω ∞ δ φ |
| ? | ¿ |
| ! | ¡ |
| $ | ¢ £ ¥ Pts  € |

# APPENDIX H: FILE LOG

| File No | Descriptions |
| --- | --- |

# APPENDIX I: REMOTE KEYS DESCRIPTION

1.ON Power On / Off. If there is no file in 01 to 99, it will run file 00.

If there is file in 01 to 99, it will display the last saved file.

2.PROG Enter the Edit Program Mode. Select 01 to 99 to enter message.

(File 00 for power on E1000 series demonstration, it can’t be edited.)

REMARK: [Edit Program Mode] The pressed key will be shown in [Command] within the message.

[In edit Program Mode] The pressed key will do some operation.

3. [Edit Program Mode] CPW Use for edit or reset password.

(Use PREV and NEXT to select (a) to (d) )

(a) UNUSE PASS 		Turn off password security.

(b) USE PASS			Turn on password security.

(c) RES PASS			Reset to factory default password “000”.

(d) CHANGE PASS	      Change to new password.

4. [Edit Program Mode] MET To select the 25-display method.

(use PREV and NEXT to select (1) to (25) method )

5. [In Edit Program Mode] PREV Select option backward.

6. [In Edit Program Mode]  NEXT  Select option forward.

7. [In Edit Program Mode] CAP Toggle between Capital letter and lower case character.

8. [Edit Program Mode] TIME to select time option.

( use PREV and NEXT to select the following options: )

(a) SET TIME			Set the current time.

(b) SET DATE			Set the current date.

(c) ALARM  			Set alarm times and interval.

e.g. C: 123M:001 means there will be 123 times of alarm signal at 1-minute interval.

(d) HOURLY ALARM	     Set the hourly alarm On / Off.

(e) 12- HOUR		     The time will be displayed in 12-hour format.

(f) 24- HOUR		     The time will be displayed in 24-hour format.

(g) ADD TIME		     Add time screen in display message.

(h) ADD DATE		     Add date screen in display message.

(i) OFF TIME		     Turn off the E1000 series at the programmed time.

(j) ON TIME		     Turn on the E1000 series at the programmed time.

(k) ADD TEMP		     Add temperature in display message. (Only available for models

has temperature function installed)

9. [Edit Program Mode] SPE select the display speed of the message.

( use PREV and NEXT to select the different speed.)

10. [Edit Program Mode] FONT select the font size of the display message.

( use PREV and NEXT to select the different font size.)

11. [Edit Program Mode] COL select the color of the display message.

( use PREV and NEXT to select the different color of the display message.)

12. [Edit Program Mode] SYM select 30 built-in symbol.

( use PREV and NEXT to select the different symbol.)

13. [In Edit Program Mode] CLR erase option.

( use PREV and NEXT to select the following options: )

(a) DEL ALL			Clear all messages.

(b) DEL MSG			Clear the current message.

(c) DEL FILES??		      Delete the specified file (01 to 99) or sequence file (S0 to S9)

(d) DEL ON/OFF 		Delete the Auto ON/ OFF timer function.

(e) DEL ALARM 		Delete the alarm function.

14. [Edit Program Mode] GRA select graphic option.

( use PREV and NEXT to select the following options: )

(a) GET GRAPH		Place the graphic in the message. Use PREV and NEXT to select

GRAPH A to GRAPH H

(b) MAKE GRAPH		Create a new graphics in location GRAPH A to GRAPH H.

(c) EDIT GRAPH		Edit GRAPH A to GRAPH P and store in one of the location of

the location of GRAPH A to GRAPH H.

REMARK: Please refer to 28. Cursor direction pointer to make and edit graphic.

15. [In Edit Program Mode]	 <=  Moving the message and command to the right.

16. [In Edit Program Mode]	 =>  Moving the message and command to the left.

REMARK:  IF YOU WANT TO DELETE MESSAGE OR COMMAND IN THE EDIT PROGRAM MODE. PLEASE USE ABOVE KEYS TO MOVE THAT MESSAGE OR COMMAND TO THE LEFT-MOST POSITION AND PRESS DEL .

IF YOU WANT TO INSERT MESSAGE OR COMMAND IN THE EDIT PROGRAM MODE. PLEASE USE ABOVE KEYS TO MOVE THAT MESSAGE OR COMMAND TO THE LEFT-MOST POSITION AND (1) PRESS INS TO INSERT. (2) TYPE IN YOUR MESSAGE OR PRESS COMMAND KEY ,IT WILL INSERT AUTOMATICALLY.

17. [In Edit Program Mode]	ALT use the alternative symbol in the keyboard.

18. [Edit Program Mode] PAU generate a pause within the programmed message.

( use PREV and NEXT to select PAUSE 1 to PAUSE 8. )

19. [Edit Program Mode] PHR select a programmed animation in the message.

( use PREV and NEXT to select ANIM 1 to ANIM 8. )

20. [Edit Program Mode] BEEP Select beep sound in the message.

( use PREV and NEXT to select BEEP 1 to BEEP 3. )

21. ENT accepts the selected option.

22. [In edit Program Mode] ESC return to the upper manual in the edit program mode.

23. [In edit Program Mode] INS refer to 15 and 16.

24. [In edit Program Mode] DEL refer to 15 and 16.

25. [Edit Program Mode] SPA insert space in the message..

26. RUN  (1) In edit program mode, press RUN to save and run the current message.

(2) In display mode, press RUN and enter 01 to 99 to select other message for display.

27. [Edit Program Mode] S please refer to the topic: Programming a sequence file.

28.   	D		E	   	F         Cursor direction pointer.  J  toggle cursor

I		J	   	K          on /off

N		O	   	P

REMARK: The direction point only work in graphic edit mode.

# APPENDIX J: ORDERING INFORMATION

Models naming convention

- E1xxxC represent Multi-color models

- E108xx represent 80 points width models

- E106xx represent 60 points width models

- E110xx represent 100 points width models

- E112xx represent 120 points width models

- E1xx1x represent outdoor models

- E1xx2x represent indoor models with temperature display

- E1xx3x represent Φ3.75 indoor models, which powered with 5V DC

| MODEL | DESCRIPTION |
| --- | --- |
| E1080C | 80 points, Multi-color, Indoor, 7.5V AC (Standard) |
| E1080R | 80 points, single red color, Indoor, 7.5V AC |
| E1081C | 80 points, Multi-color, Outdoor, 7.5V AC |
| E1082C | 80 points, Multi-color, Indoor, Temperature displaying, 7.5V AC |
| E1083C | 80 points, Multi-color, Indoor, Φ3.75 LED module, 5V DC |
| E1060C | 60 points, Multi-color, Indoor, 7.5V AC |
| E1060R | 60 points, single red color, Indoor, 7.5V AC |
| E1064C | 64 points, Multi-color, Indoor, 7.5V AC |
| E1064R | 64 points, single red color, Indoor, 7.5V AC |
| E1100C | 100 points, Multi-color, Indoor, 7.5V AC |
| E1100R | 100 points, single red color, Indoor, 7.5V AC |
| E1120C | 120 points, Multi-color, Indoor, 7.5V AC |
| E1120R | 120 points, single red color, Indoor, 7.5V AC |

# TERMS AND CONDITIONS

Customer must produce this warranty card in conjunction with the original receipt of purchase when  warranty service is requested during warranty period.

Warranty: The products are warranted against defects in materials and workmanship. If we receive notice of such defects during the warranty period , we shall either repair or replace the products which prove to be defective.

Limitation of Warranty: Warranty shall not apply to defects resulting from:

- Improper or inadequate maintenance by Buyers;

- Unauthorized modification or misuse;

- Operation outside of the environmental specifications for the products, or

- Improper site preparation and maintenance.

# TROUBLE SHOOTING

No display            	-Check if the power socket located at the back of the sign is connected.

-Check if the AC power plug is plugged into the wall outlet.

-Check if the main power of the AC outlet is normal.

-Check if the auto on/auto off time setting is correct.

Not response to

remote control	-Check if the batteries of the remote control still have enough energy. If  user is not sure, please replace all the batteries.

-Check if the batteries of the remote control are placed at right polarity. Please follow the markings in the battery compartment.

-Check if there is anything between the remote and the sign blocking the  remote signal.

No display but

response to the

remote control      	-Check if the auto on/auto off time setting is correct.

-Check if the data in the files are valid.

-If you are not sure, you can enter the edit mode and press ‘CLR’ key of the remote control, select ‘DEL ALL’ to erase all data, this will reset the sign to display the default demo messages.

If the above procedures do not work, please contact your dealer.