# Connecting your PC to a sign

In this configuration, one or more signs are connected to existing Local Area Network(LAN):

# Setting for Ethernet Adapters:

Before can transmit messages to Signs by Local Area Network, the Ethernet Adapters must be setting correctly.

To set Ethernet Adapters ,open the NetJetSetting program, it will appear a window as follows:

## Group Parameter Configuration

All settings in Group Parameter Configuration are to be download to Ethernet Adapters.

- Device IP - Specify the 4-node Internet Protocol address for this Ethernet Adapters. See your network administrator if you do not know this address. The default setting is 192.1.1.12 .

- Device Port – The IP port you want to set. The default value is 5005.

- Gateway – Specify the 4-node Internet Protocol address for Gateway of Local Area Network. If PC with E2000TP for Windows messaging software is in the same sub-net with Ethernet Adapters connected to Signs ,it is not useful . Whereas, it should be specified. For example, (Assuming the sub-net mask is 255.255.255.0), the IP for PC with E2000TP for Windows messaging software is 192.168.1.150, IP for Ethernet Adapters is 192.168.2.50,  and  the one of IP for Gateway of Local Area Network is 192.168.1.10 , so the PC with E2000TP for Windows messaging software is not in the same sub-net with Ethernet Adapters , so you must specify the Gateway to 192.168.1.10 . See your network administrator if you do not know this address. The default setting is 192.1.1.254 .

- Subnet mask – If your Local Area Network is Class C network ,the sub-net mask is 255.255.255.0 , if your Local Area Network is Class B network ,the sub-net mask is 255.255.0.0 , if your Local Area Network is Class A network ,the sub-net mask is 255.0.0.0. See your network administrator if you do not know this address.

- Baudrate – When Ethernet Adapters receive messages from PC with E2000TP for Windows messaging software ,it will transmit these messages to Signs according to this speed .So it should select 9600. The default value is 9600.

- Parity ,Databits ,Stopbits – It define the data formats .It is fixed.

## Group Setup Mode

User can download all settings in group Parameter Configuration to Ethernet Adapters by com port or by Local Area Network(LAN).

- By com port:

First, user must use 232 cable to connect Ethernet Adapter to PC COM port(Assuming COM2).

Then, select COM2 in Port, and select 9600 in Baud .

At last click button Config by Serial to download all settings to Ethernet Adapter.

Note: If user want to download the settings to Ethernet Adapter by com port, user MUST turn off the power for Ethernet Adapter for several seconds at first and then continue the another operations.

- By Local Area Network(LAN):

First, connected the Ethernet Adapter to Local Area Network.

Second, typed the IP address for Ethernet Adapter in Local IP . This IP address is the current IP address for Ethernet Adapter .If it is the first time to set, this IP is 192.1.1.12.

Third, typed the IP port in Device Port. If it is the first time to set , this IP port is 5005.

At last, click button Config by Network to download all settings to Ethernet Adapter.

Note: Because the default IP address for Ethernet adapter is 192.1.1.12, If it is the first time to change the settings ,user must change the PC’s IP to the same sub-net with Ethernet Adapter. In other words, user must change the PC’s IP to 192.1.1.X, and then change the Ethernet Adapter’s settings. If you don’t know how to do it, please ask your network administrator.