# Operation Manual for ExcelPoll Quemetco Application

# Document Filename Specification

Document filename is [/tmp/in/input.doc]

# Revision History

| Version | Date | Who? | Comments |
| --- | --- | --- | --- |
| 0.1 | 12/14/09 | D.Hlavac | Initial Revision. |

# Overview

This document describes the operation of the ExcelPoll application. This application polls a selected excel spreadsheet and display the cells on a multi-line sign.

# Installation

To install, simply unzip the ExcelPoll.zip file to any location on the PC from the distribution media.

Since this application uses excel files natively, there is no need to have Microsoft Office installed on the machine. This application supports any Excel file (.xls).

This also means that the file must be SAVED for any changes to occur on the signs.

To run the program, double-click on ExcelPoll.exe. The following screen will appear:

The application automatically polls the selected file every n seconds, then runs the sign by showing each sheet of the workbook.

# Workbook Format

The workbook must be in the following format in order for the polling to work correctly:

- The number of sheets equals the number of screens to show.

- Cell D1 must contain the width, in characters, of the signs being controlled for this sheet. This is not used as this is a custom application and can be set to any number.

- Cell D2 must contain the number of lines to display for that sheet.

- Cell D3 must contain the number of seconds to display this screen/sheet.

- Rows [4….n] must contain each line of text to display, up to the number given in A2. Each row’s cell (An..Dn) must contain the text to display, which is combined left to right to form the line. Each cell can contain up to 6 characters, which are combined to send to a single line on the display. Up to 4 cells, 6 characters each, or 24 characters.

- An example workbook is included under Quemetco Display.xls.

- Colors have no effect.

- Formatting does have an effect for left, right, and center horizontal justifications.

- Cells should only be formatted for general or text. Any other combinations may produce undesired results due to translations.

- Data in other cells have no effect.

If you have errors when the application polls the workbook, make sure the number of sheets matches the number of screens. Normally Excel creates three blank sheets. You must delete unused sheets.

# Configuration

To configure the application:

- Pause the application by clicking “Pause”

- Select the filename to poll. This can be on a network or local.

- Select the amount of time to poll this file.

- Enter the ip address of the sign.

- Resume the application by clicking “Go”.

Configuration is persistent and is saved automatically.

# Operation

Operation automatically starts when the poll interval has elapsed. In order to start it up right away, or for a manual poll, click Poll Now.