Wzp2006 is upgraded to support custom fonts using with MCS protocol.

WARNING: Do not turn off the power while updating OS. The sign will automatically restart after it finish.

Make sure the PC have been linked to the E2000 sign as below:

E2000 sign

PC

cable

Run Wzpsoft.exe to lunch the Wzp2006 LED Center.

Choose menu Help | Content to learn more about updating OS.

Choose menu Comm | Install OS & Fonts.

Make sure that both “I want to update OS on the sign” and “I want to update fonts on the sign” is checked, and then click Browse.

Select the new OS file “lmos.sys” then click Open.

Click Add.

Select the font files “AR24.lmf” “TM24.lmf” (hold down <Ctrl> for multi-selection), and then click Open.

Now the new fonts AR24 and TM24 are in the fonts list, indexing from 5 to 6. Click OK.

When these progresses finished, the sign will display “Writing system files…”

Writing system files…

WARNING: Do not turn off the power while writing system files. The sign will automatically restart after it finish. Only the JTAG connection is available when the sign is damaged by improper updating operation.

Run the QuickMCS.exe, edit text A as below:

You can use either ^F5 or ^F$TM24$ to specify using font TM24.

You can use either ^F6 or ^F$AR24$ to specify using font AR24.

Select menu Comm | Send Text A.

The sign will give a short beep and then display text A with new fonts TM24 and AR24.

TM24 AR24

(End)