Wzp2006 is upgraded to support custom fonts using with MCS protocol.

WARNING: Do not turn off the power while updating firmware. The sign will automatically restart after it finish.

Make sure the PC have been linked to the WZP sign as below:

WZP sign

PC

cable

Run Wzpsoft.exe to launch the Wzp sign software. (Note: If wants to get “Update Firmware & MCS Font” function, user needs to modify “config.ini” file.)

Choose menu Help | Content to learn more about updating firmware.

Choose menu Comm | Update Firmware & MCS Fonts.

Select “I want to update Firmware on the sign” and then click Browse button.

Select the new OS file “lmos.sys” then click Open.

Click OK button.

Select “I want to update MCS fonts on the sign”

Click Add.

Select the font files “AR24.lmf” “TM24.lmf” (hold down <Ctrl> for multi-selection), and then click Open.

Now the new fonts AR24 and TM24 are in the fonts list, indexing from 7 to 8. Click OK.

When these progresses finished, the sign will display “Writing system files…”

Writing system files…

WARNING: Do not turn off the power while writing system files. The sign will automatically restart after it finish. Only the JTAG connection is available when the sign is damaged by improper updating operation.

Run the QuickMCS.exe, edit text A as below:

You can use either ^F5 or ^F$TM24$ to specify using font TM24.

You can use either ^F6 or ^F$AR24$ to specify using font AR24.

Select menu Comm | Send Text A.

The sign will give a short beep and then display text A with new fonts TM24 and AR24.

TM24 AR24

(End)